F.3 Chapter 3 Solutions

3.1

|  | N-Type | P-Type |
| --- | --- | --- |
| Gate=1 | closed | open |
| Gate=0 | open | closed |

3.3 There can be 16 different two input logic functions.

3.5

| A | B | C |  | OUT |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 |  | 1 |
| 0 | 0 | 1 |  | 0 |
| 0 | 1 | 0 |  | 1 |
| 0 | 1 | 1 |  | 0 |
| 1 | 0 | 0 |  | 1 |
| 1 | 0 | 1 |  | 0 |
| 1 | 1 | 0 |  | 0 |
| 1 | 1 | 1 |  | 0 |

3.7 There is short circuit (path from Power to Ground) when either A = 1 and B = 0 or A = 0 and B = 1.

  - The circuit is floating when A = 0 and creates a short-circuit when A = 1, which is guaranteed to burn out the power supply.

  - When A = 0, Out = 3.3 V
      When A = 1, Out is floating, but power and ground are shorted, which will result in very high current and possible breakdown.

  - *NOTE: This problem was mistakenly included in Chapter 3 when it should belong in Chapter 5. See ERRATA for more information*
OUT = 1 signifies that the instruction is a branch instruction, and the branch will be taken. OUT = 0 means otherwise.

3.15

| A | B |  | NOT(NOT(A) OR NOT(B)) |
| --- | --- | --- | --- |
| 0 | 0 |  | 0 |
| 0 | 1 |  | 0 |
| 1 | 0 |  | 0 |
| 1 | 1 |  | 1 |

AND gate has the same truth table.

3.17 a. Three input And-Gate

Three input OR-Gate

b. (1) A = 1, B = 0, C = 0.

AND Gate

OR Gate

b. (2) A = 0, B = 0, C = 0

AND Gate

OR Gate

b. (3) A = 1, B = 1, C = 1

AND Gate

OR Gate

3.19 A five input decoder will have 32 output lines.

3.21

| Cin | 1 | 1 | 1 | 0 |
| --- | --- | --- | --- | --- |
| A | 0 | 1 | 1 | 1 |
| B | 1 | 0 | 1 | 1 |
| S | 0 | 0 | 1 | 0 |
| Cout | 1 | 1 | 1 | 1 |

A = 7, B = 11, A + B = 18.

In the above calculation, the result (S) is 2 !! This is because 18 is too large a number to be represented in 4 bits. Hence there is an overflow - Cout[3] = 1.

3.23 (a) The truth table will have 16 rows. Here is the truth table for Z = XOR (A, B, C, D). Any circuit with at least seven input combinations generating 1s at the output will work.

| A | B | C | D |  | Z |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |  | 0 |
| 0 | 0 | 0 | 1 |  | 1 |
| 0 | 0 | 1 | 0 |  | 1 |
| 0 | 0 | 1 | 1 |  | 0 |
| 0 | 1 | 0 | 0 |  | 1 |
| 0 | 1 | 0 | 1 |  | 0 |
| 0 | 1 | 1 | 0 |  | 0 |
| 0 | 1 | 1 | 1 |  | 1 |
| 1 | 0 | 0 | 0 |  | 1 |
| 1 | 0 | 0 | 1 |  | 0 |
| 1 | 0 | 1 | 0 |  | 0 |
| 1 | 0 | 1 | 1 |  | 1 |
| 1 | 1 | 0 | 0 |  | 0 |
| 1 | 1 | 0 | 1 |  | 1 |
| 1 | 1 | 1 | 0 |  | 1 |
| 1 | 1 | 1 | 1 |  | 0 |

Z = XOR (A,B,C,D)

(b)

A B C D

Z

3.25 Figure 3.36 is a simple combinational circuit. The output value depends ONLY on the input values as they currently exist. Figure 3.37 is an R-S Latch. This is an example of a logic circuit that can store information. That is, if A, B are both 1, the value of D depends on which of the two (A or B) was 0 most recently.

3.27 2 * 214 = 215 = 32768 nibbles

3.28

A.

B.

3.29

| A | B | C |  | Z |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 |  | 0 |
| 0 | 0 | 1 |  | 0 |
| 0 | 1 | 0 |  | 0 |
| 0 | 1 | 1 |  | 0 |
| 1 | 0 | 0 |  | 0 |
| 1 | 0 | 1 |  | 0 |
| 1 | 1 | 0 |  | 0 |
| 1 | 1 | 1 |  | 0 |

3.31	(a) 3 gate delays

3.31	(b) 3 gate delays

3.31	(c) 3*4 = 12 gate delays

3.31	(d) 3*32 = 96 gate delays

3.32	For the OR gate generating the Si, connect the 4 outputs: 001, 010, 100, and 111 from the decoder to the OR gate. Connect the remaining 2 inputs of the OR gate to 0.

For the OR gate generating the Ci+1, connect the 4 outputs: 011, 101, 110, and 111 from the decoder to the OR gate. Connect the remaining 2 inputs of the OR gate to 0.

3.33(a) When S=0, Z = A

3.33(b) When S=1, Z retains its previous value. 3.33(c) Yes; the circuit is a storage element.

3.35 No. The original value cannot be recovered once a new value is written into a register.

3.36 a)ABGEL

00010

01001

10100

11010

b) G = AB' , L = A'B , E = A'B' + AB

c) Y = G[3] + E[3]G[2] + E[3]E[2]G[1] + E[3]E[2]E[1]G[0]

3.37. 8 * (2^3) = 64 bytes

3.39.(a) To read the 4th memory location, A[1,0] = 11, WE = 0

3.39.(b) A total of 6 address lines are required for a memory with 60 locations. The addressability of the memory will remain unchanged.

3.39.(c) A program counter of width 6 can address 2^6 = 64 locations. So without changing the width of the program counter, 64-60 = 4 more locations can be added to the memory.

3.41 Total bits of storage = 2^22 * 3 = 12582912

3.43 There are a total of four possible states in this lock. Any other state can be expressed as one of states A, B, C or D. For example, the state performed one correct followed by one incorrect operation is nothing but state A as the incorrect operation reset the lock.

  - No. An arc is needed between the two states.

    - Game in Progress:

Texas *	Oklahoma

Fouls:4	Fouls: 4

73	68

First Half 7:38

Shot Clock : 14

    - Texas Win:

Texas *	Oklahoma

Fouls:10	Fouls: 10

85	70

Second Half 0:00

Shot Clock : 0

    - Oklahoma Win:

Texas *	Oklahoma

Fouls:10	Fouls: 10

81	90

First Half 7:38

Shot Clock : 0

No coins

N

Q

5

D/Q

Q

N

30

N

D

D

35+

ch

N

D

35

10

Q

D

Q

25

N

D

D

15

35+

ch

N

20

N

Q	Q

35+

ch

35+

ch

3.47

3.49

a)

| S1 | S0 | X |  | D1 | D0 | Z |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 |  | 0 | 0 | 0 |
| 0 | 0 | 1 |  | 0 | 0 | 0 |
| 0 | 1 | 0 |  | 0 | 0 | 1 |
| 0 | 1 | 1 |  | 1 | 0 | 1 |
| 1 | 0 | 0 |  | 1 | 1 | 1 |
| 1 | 0 | 1 |  | 1 | 1 | 1 |
| 1 | 1 | 0 |  | 1 | 0 | 1 |
| 1 | 1 | 1 |  | 1 | 0 | 1 |

0/1

0

00

01

0/1

1

11

10

0/1

b)

3.50

NOT: A NAND A => NOT A
AND: (A NAND B) NAND (A NAND B) = NOT (A NAND B) => A AND B

3.51 	Flip-Flop

3.52 	A=1,B=0

3.53  





















	The circuit is a decrementing 3-bit counter.

3.55

| a | b | c |  | Y | Z |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 |  | 0 | 0 |
| 0 | 0 | 1 |  | 1 | 0 |
| 0 | 1 | 0 |  | 1 | 1 |
| 0 | 1 | 1 |  | 0 | 1 |
| 1 | 0 | 0 |  | 1 | 1 |
| 1 | 0 | 1 |  | 0 | 1 |
| 1 | 1 | 0 |  | 0 | 0 |
| 1 | 1 | 1 |  | 1 | 0 |

3.57

3.59

| S1 | S0 | X |  | Z | S1’ | S0’ |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 |  | 1 | 0 | 0 |
| 0 | 0 | 1 |  | 1 | 0 | 1 |
| 0 | 1 | 0 |  | 0 | 1 | 0 |
| 0 | 1 | 1 |  | 0 | 0 | 0 |
| 1 | 0 | 0 |  | 0 | 0 | 1 |
| 1 | 0 | 1 |  | 0 | 1 | 0 |
| 1 | 1 | 0 |  | 0 | 0 | 0 |
| 1 | 1 | 1 |  | 0 | 0 | 0 |

3.61